Home > Products > Coils＆Inductors > PK; [ Back ].

PK. PEAKING COIL Shape and size : Features : - Small size radial lead type . - Small mounting space required . - Excellent characteristics for High Q . - Minimized distributed capacitance hence high SRF . - Special lead wire construction prevents open circuit failures . - Covered with PVC sleeve or UL tube . - Can be taped for auto insertion . Ordering information : (1) Type : Peaking coils . (2) Style : Core D=6 mm ; L=8 mm (3)Inductance : Example : 503 is 50 mH (4)Inductance tolerance : J :5% ; K :10% ; M: 20% . (5)PVC Sleeve or UL tube (6)Sleeve color code . Characteristics : - Inductance range : 1.0uH to 100000uH . - Current rating: Based on temperature rise not to exceed 20oC . - Operating temperature : -20oC to 80oC. - Terminal strength : 0.5 Kg min . - Test equipment : For PK0406-1R0K to 102K, L tested by HP4342A Q Meter For PK0406-122K to 253K, PK0608, PK0810 & PK1012, L tested by LCR meter @ 1KHz . Q tested by HP 4342A Q meter . DCR tested by Milliohm meter . SRF tested by HM 9461 L-SRF meter . Applications : - TVs and Audio equipment . - Telecommunication devices . - Buzzers and Alarm systems . - Switching Power Supplies. - Systems requiring Band and High Q. - Other noise filters ...