BA1

HANDOUK

►FEATURES

- Most small size, Low Cost

- SPDT configurations

- PCB mounting, DIP package

►CONTACT DATA

| ## Contact Arrangement | 1 C（1 Form C） |
| --- | --- |
| Contact Material | Ag + Au |
| Contact Rating (resistive) | 0.5A 125VAC, 1A 24VDC |
| Max Switching Voltage | 125VAC / 60VDC |
| Max Switching Current | 1A |
| Max Switching Power | 62.5VA / 30W |
| Contact Resistance | 100mΩ Max at 6VDC 1A |
| Life Expectancy Electrical | 100,000 Operations ( at30 Operations / minute) |

►GENERAL DATA

| Resistance |  |  | 100MΩ Min. at 500VDC |
| --- | --- | --- | --- |
| Dielectric Strength |  | Open Contacts | 400VAC (for one minute) |
|  |  | Contacts and coil | 1000VAC (for one minute) |
| Operate Time |  |  | 5ms Max |
| Release Time |  |  | 5ms Max |
| Temperature Range |  |  | -30C ~ +70C |
| Shock Resistance | Operating Extremes |  | 10G |
|  | Damage Limits |  | 100G |
| Vibration Resistance |  |  | 10-55Hz Double amplitude of 2.0mm |
| Humidity |  |  | 35%-85%RH |
| Weight |  |  | Approx 2.2g |
| Safety Standard |  |  | UL E197546 |

►COIL DATA

| Coil voltage (VDC) | Coil Resistance at 20C |  |  | Pick-up Voltage Max |  |  | Drop-out Voltage Min | Applicable Voltage Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.36W | 0.2W | 0.15W | 0.36W | 0.2W | 0.15W |  |  |
| 3 | 25 | 45 | 60 | 2.1 | 2.25 | 2.4 | 0.3 | 3.9 |
| 5 | 69.4 | 125 | 167 | 3.5 | 3.75 | 4.0 | 0.5 | 6.5 |
| 6 | 100 | 180 | 240 | 4.2 | 4.5 | 4.8 | 0.6 | 7.8 |
| 9 | 225 | 405 | 540 | 6.3 | 6.75 | 7.2 | 0.9 | 11.7 |
| 12 | 400 | 720 | 960 | 8.4 | 9 | 9.6 | 1.2 | 15.6 |
| 24 | 1600 | 2880 | 3840 | 16.8 | 18 | 19.2 | 2.4 | 31.2 |

►ORDERING INFORMATION

►DIMENSIONS  (Unit: mm)