BC2

HANDOUK

■ FEATURES

* High dielectric withstand of 2,000VAC (Between coil and contacts)
* High contact capacity:2A 30VDC
* Surge withstand voltage up to 2,500VAC (Between coil and contacts)
* SMT type available
* Bifurcated contacts

* Single side stable and & latching type available

■ CONTACT DATA

| Contact Arrangement |  | DPDT (2 Form C) |
| --- | --- | --- |
| Initial Contact Resistance |  | 50m ohm (Max) |
| Contact Material |  | AgNi + Au plated |
| Rated load (Res. Load) |  | 2A 30VDC, 0.5A 125VAC |
| Max. switching power |  | 60W, 62.5VA |
| Max. switching Voltage |  | 220VDC, 250VAC |
| Max. switching current |  | 2A |
| Min. Applicable Load |  | 10mV 10 micro A |
| Life Expectancy | Electrical | 1 x 105 ops (2A 30VDC) 5 x 105 ops (1A 30VDC) 1 x 105 ops (0.5A 125VAC) |
|  | Mechanical | 1 x 108 ops (at 0.5A 125VAC |

■ GENERAL DATA

| Initial Insulation resistance |  |  |  | 1000M ohm 500VDC |
| --- | --- | --- | --- | --- |
| Dielectric Strength between | Coil and contacts |  |  | 2,000VAC 1min. |
|  | Open contacts |  |  | 1,000VAC 1min. |
|  | Contact sets |  |  | 1,500Vrms, 1min |
| Surge withstand voltage |  |  |  | Between open contacts(10x160µs) :1,500VAC (FCC part 68) Between coil and contacts(2x10µs) : 2,500VAC (Bellcore) |
| Operate time (nominal voltage) |  |  |  | Max. 4ms |
| Release time (nominal voltage) |  |  |  | Max. 4ms |
| Ambient temperature |  |  |  | -40°C~+85°C |
| Humidity |  |  |  | 35% to 85% R.H. |
| Vibration resistance |  |  | Functional | 10 to 55Hz 3.3mm DA |
|  |  |  | Destructive | 10 to 55Hz 5mm DA |
| Shock resistance |  | Functional |  | 750m/s2(75g) |
|  |  | Destructive |  | 1,000m/s2(100g) |
| Termination |  |  |  | DIP, SMT |
| Unit weight |  |  |  | Approx. 2g |
| Construction |  |  |  | Sealed IP67 |

■ COIL

| Coil power | Single side stable | 140mW |
| --- | --- | --- |
|  | Latching 1 coil | 100mW |
|  | Latching 2 coil | 200mW |
| Temperature Rise |  | Max. 50°C |
| Coil Voltage |  | See coil data table |

■ SAFETY APPROVAL RATING

| UL&CUR | 2A 30VDC 0.5A 125VAC |
| --- | --- |

■ COIL DATA
Single side stable                                            standard(140mW) 20°C

| Norminal Voltage (VDC) | Norminal current (mA) | Pick-up voltage VDC (Max.) | Drop-out voltag VDC (Min.) | Coil resistance Ω (10%) | Nominal Power (mW) | allowable Voltage VDC(Max.) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.5 | 93.8 | 1.13 | 0.15 | 16 | 140 | 2.2 |
| 3 | 46.7 | 2.25 | 0.3 | 64.3 | 140 | 4.5 |
| 4.5 | 31 | 3.38 | 0.45 | 145 | 140 | 6.7 |
| 5 | 28.1 | 3.75 | 0.5 | 178 | 140 | 7.5 |
| 6 | 23.3 | 4.5 | 0.6 | 257 | 140 | 9 |
| 9 | 15.5 | 6.75 | 0.9 | 579 | 140 | 13.5 |
| 12 | 11.7 | 9 | 1.2 | 1,028 | 140 | 18 |
| 24 | 5.8 | 18 | 2.4 | 4,114 | 140 | 36 |
| 48 | 5.6 | 36 | 4.8 | 8,533 | 270 | 57.6 |

Latching (1 coil)                                                 standard(100mW) 20°C

| Norminal Voltage (VDC) | Norminal current (mA) | Set/Reset voltage VDC (Max.) | Coil resistance Ω (10%) | Nominal Power (mW) | allowable Voltage VDC(Max.) |
| --- | --- | --- | --- | --- | --- |
| 1.5-L1 | 66.7 | 1.13 | 22.5 | 100 | 2.7 |
| 3-L1 | 33.3 | 2.25 | 90 | 100 | 5.4 |
| 4.5-L1 | 22.2 | 3.38 | 203 | 100 | 8.1 |
| 5-L1 | 20 | 3.75 | 250 | 100 | 9 |
| 6-L1 | 16.7 | 4.5 | 360 | 100 | 10.8 |
| 9-L1 | 11.1 | 6.75 | 810 | 100 | 16.2 |
| 12-L1 | 8.3 | 9 | 1,440 | 100 | 21.6 |
| 24-L1 | 4.2 | 18 | 5,760 | 100 | 43.2 |

Latching (2 coil)                                                standard(200mW) 20°C

| Norminal Voltage (VDC) | Norminal current (mA) | Set/Reset voltage VDC (Max.) | Coil resistance Ω (10%) | Nominal Power (mW) | allowable Voltage VDC(Max.) |
| --- | --- | --- | --- | --- | --- |
| 1.5-L2 | 133.9 | 1.13 | 11.2 | 200 | 2.7 |
| 3-L2 | 66.7 | 2.25 | 45 | 200 | 5.4 |
| 4.5-L2 | 44.5 | 3.38 | 101 | 200 | 6.7 |
| 5-L2 | 40 | 3.75 | 125 | 200 | 7.5 |
| 6-L2 | 33.3 | 4.5 | 180 | 200 | 9.0 |
| 9-L2 | 22.2 | 6.75 | 405 | 200 | 13.5 |
| 12-L2 | 16.7 | 9 | 720 | 200 | 18 |
| 24-L2 | 8.3 | 18 | 2,880 | 200 | 36 |

■ ORDERING INFORMATION

■ CHARACTERISTICS CURVE

■ OVERALL AND MOUNTING DIMENSIONS