HA1

HANDOUK

■ FEATURES

● 7A, 10A Switching capability.

● Subminiature.

● Standard PC Layout.

■ CONTACT DATA

| Contact Form |  | 1A | 1C |
| --- | --- | --- | --- |
| Contact Material |  | Ag Alloy | AgSnO2 |
| Contact Ratings |  | 10A 240VAC 12A 120VAC / 24VDC | 7A 240VAC 10A 120VAC / 24VDC |
| Max Switching Voltage |  | 250VAC / 30VDC |  |
| Max Switching Current |  | 15A |  |
| Max Switching Power |  | 2,770VA / 240W |  |
| Contact Resistance |  | 100mΩ Max at 6VDC 1A |  |
| Life Expectancy | Electrical | 100,000operations at 30operations/minute) |  |
|  | Mechanical | 10,000,000operations |  |

■ GENERAL DATA

| Insulation Resistance |  | 100M Ω Min. at 500VDC |
| --- | --- | --- |
| Dielectric Strength | Open Contacts | 750VAC (For one minute) |
|  | Contacts and Coil | 1,500VAC (For one minute) |
| Operate Time |  | 10ms |
| Release Time |  | 5ms |
| Temperature Range |  | -40℃ ~ 70℃ |
| Shock Resistance | Operate Extremes | 10G |
|  | Damage Limits | 100G |
| Vibration Resistance |  | 10 ~ 50Hz 1.5mm |
| Humidity |  | 40 ~ 85% |
| Weight |  | Approx 10g |
| Safety Standard |  | TUV R50054834 |
|  |  | UL197546 |

■ COIL DATA

| Nominal Voltage (VDC) | Coil Resistance At 20 ±10% Ω |  | Max Operating Voltage VDC | Min Release Voltage VDC | Max Applicable Voltage VDC |
| --- | --- | --- | --- | --- | --- |
|  | 0.36W | 0.45W |  |  |  |
| 3 | 25 Ω |  | 2.25 | 0.3 | 3.9 |
| 5 | 70 Ω |  | 3.75 | 0.5 | 6.5 |
| 6 | 100 Ω |  | 4.5 | 0.6 | 7.8 |
| 9 | 225 Ω |  | 6.75 | 0.9 | 11.7 |
| 12 | 400 Ω |  | 9 | 1.2 | 15.6 |
| 18 | 900 Ω |  | 13.5 | 1.8 | 23.4 |
| 24 | 1,600 Ω |  | 18 | 2.4 | 31.2 |
| 48 | 6,400 Ω±15% | 5,100 Ω±15% | 36 | 4.8 | 62.4 |

■ ORDERING INFORMATION

■ OVERALL AND MOUNTING DIMENSIONS