HL1

HANDOUK

►FEATURES

● 1 Form A, I Form C, 2 Form A,

2 Form C configurations.

● 5A, 10A, 16A contact ratings.

● Sealed types.

Size (L x W x H)

29 x 12.7 x 20mm

►CONTACT DATA

| Contact Form |  | 1C (10A) | 1CH (16A) | 1CT (16A) | 2C (5A) | 2CM (8A) |
| --- | --- | --- | --- | --- | --- | --- |
| Contact Ratings |  | 10A 250VAC 30VDC | 16A 250VAC 30VDC | 16A 250VAC 30VDC | 5A 250VAC 30VDC | 8A 250VAC 30VDC |
| Contact Material |  | Ag alloy |  |  |  |  |
| Max. Switching Voltage |  | 250VAC / 30VDC |  |  |  |  |
| Max. Switching Current |  | 12A | 20A | 20A | 8A | 8A |
| Max. Switching Power |  | 2500VA/300W |  |  |  |  |
| Contact Resistance |  | 100mΩ at 6VDC 1A |  |  |  |  |
| Life Expectancy | Electrical | 100,000 Operations (at 30 ops a minute) |  |  |  |  |
|  | Mechanical | 10,000,000 Operations |  |  |  |  |

►OPERATION CONDITION

| Insulation Resistance |  |  | 100MΩ min (at 500VDC) |
| --- | --- | --- | --- |
| Dielectric Strangth | Between open Contacts |  | 1,000VAC (For one minute) |
|  | Between coil & Contacts |  | 5,000VAC (For one minute) |
| Operate time |  |  | 20ms |
| Release Time |  |  | 10ms |
| Shock resistance | Operating Extremes |  | 10G |
|  | Damage Limits |  | 100G |
| Vibration resistance |  |  | 10~55Hz Double amplitude |
| Humidity |  |  | 40~85% |
| Temperature Range |  |  | -30 ~ 70℃(-40~158°F) |
| Weight |  |  | Approx 14g |
| Safety Standard |  | UL | E197546 |
|  |  | TUV | R50056430 |

►COIL PARAMETER

| Nominal Voltage (VDC) | Coil Resistance at 20 C (Ω) |  | Max Operate Voltage | Min. Release Voltage | Max. Applicable Voltage |
| --- | --- | --- | --- | --- | --- |
|  | 0.54W | 0.72W |  |  |  |
| 3 | 17 | 13 | 2.4 | 0.15 | 3.9 |
| 5 | 46 | 35 | 4 | 0.25 | 6.5 |
| 6 | 67 | 50 | 4.8 | 0.3 | 7.8 |
| 9 | 150 | 110 | 7.2 | 0.45 | 11.7 |
| 12 | 270 | 200 | 9.6 | 0.6 | 15.6 |
| 24 | 1050 | 800 | 19.2 | 1.2 | 31.2 |
| 48 | 4,250±15% | 3,200 | 38.4 | 2.4 | 62.4 |
| 60 | 6,800 ±15% |  | 48 | 3.0 | 78 |
| 100 | 18,8600±15% |  | 80 | 5.0 | 130 |

►ORDERING INFORMATION

►DIMENSIONS  (Unit: MM)

HL1-1C

HL1-1CT

HL1-2C