ST

HANDOUK

►FEATURES

►CHARACTERISTICS ● Semi-multi power supply voltage ● Exclusive C-MOS IC permits miniaturization and assures high performance ● High repeat accuracy : ±5% including the initial value ● Short resetting time : 0.1 second max. including the time when the timer resets in operation ● Large transparent time setting knob facilitates time setting ● Equipped with a LED power-ON indicator ● Environmental friendly product (RoHs compliant) ● Outline Dimensions: 28.0 x 21.5 x 52.0mm

| Accuracy of operating time(Max) |  | ±5% |
| --- | --- | --- |
| Setting error (Max.)1) |  | ±10%±50ms |
| Reset time (Max.) |  | 0.1ms(Including halfway reset |
| Reset voltage (Max.) |  | 10% of rated supply voltage |
| Influence of Voltage (Max.)1) |  | ±2% |
| Influence of temperature (Max.)1) |  | ±2% |
| Insulation resistance (Min.) |  | 100M ohm (at 500VDC) |
| Dielectric Strength Between | contacts | 1,500VAC 1min |
|  | circuits | 1,500VAC 1min |
|  | contact sets | 1,000VAC 1min |
| Vibration resistance | Destruction | 10 to 55Hz, 0.75mm single amplitude |
|  | Malfunction | 10 to 55Hz, 0.50mm single amplitude |
| Shock resistance | Destruction | 1,000 m/s2 (100g) |
|  | Malfunction | 100 m/s2 (10g) |
| Ambient temperature | Operating | -10°C to 50°C (with no icing) |
|  | Storage | -25°C to 65°C (with no icing) |
| Ambient humidity |  | 35% to 85% RH |
| Malchanical life (Min) |  | 1x107 ops (under no load at 1,800 ops/h) |
| Electrical life (Min.) |  | ST-2 : 1x105 ops (7A 250VAC/30VDC. Res. load) ST-4 : 1x105 ops (5A 250VAC/30VDC. Res. load) |
| Enclosure rating |  | IP40 |
| Weight |  | 50g |

Notes: 1) Add ±10ms to above value for the 1s range model

►TIME RANGE

| Code | Rated time | Time setting range | Code | Rated time | Time setting range |
| --- | --- | --- | --- | --- | --- |
| A | 1 s | 0.1 to 1 s | G | 5 min | 0.2 to 5 min |
| B | 5 s | 0.2 to 5 s | H | 10 min | 0.5 to 10 min |
| C | 10 s | 0.5 to 10 s | I | 30 min | 1 to 30 min |
| D | 30 s | 1.0 to 30 s | J | 60 min | 2 to 60 min |
| E | 60 s | 2.0 to 60 s | K | 3 h | 0.1 to 3 h |

►ORDERING INFORMATION

|  | ST-2 | A | 24VDC | XXX |
| --- | --- | --- | --- | --- |
| Type | ST-2 : 7A 250VAC/30VDC, 2pole ST-4 : 5A 250VAC/30VDC, 4pole |  |  |  |
| Timing range | A to L (Show in the timing range table) |  |  |  |
| Control voltages | 12, 24, 48VDC, 110, 220, 240VAC |  |  |  |
| Customer special code | Only for special requirements |  |  |  |

►OUTLINE DEMENSIONS